| STEP 1 | STEP 2 | STEP 3 | STEP 4 |
| --- | --- | --- | --- |
| Remove the four M4 retaining screws from the front housing. | Lift the heat module clear, keeping the ribbon cable attached. | Disconnect the ribbon cable at the board connector. | Inspect the thermal pad for damage or displacement. |
| STEP 5 | STEP 6 | STEP 7 | STEP 8 |
| Position new module; align the four mounting holes. | Reconnect the ribbon cable — ensure the latch clicks. | Lower the module onto the housing and hand-start screws. | Torque screws to 1.2 Nm in a cross pattern. |